Floorplan

- Initialize Floorplan:

- initialize_floorplan -control_type core -shape L -orientation N -side_ratio {7 4 1 1}     -core_offset{15} - flip_first_row true -coincident_boundary true

- initialize_floorplan -core_utilization 0.7 -shape U -orientation N -side_ratio {2 1 1 1 1 2} -core_offset {15} -coincident_boundary true

- Rectilinear Floorplan

- initialize_floorplan -control_type die -core_offset 100 -boundary { {0 0} {0 3000} {1000 3000} {1000 2000} {2000 2000} {2000 3000} {3000 3000} {3000 0} {2000 0} {2000 1000} {1000 1000} {1000 0} }

- set_constraint_mapping_file floorplan/mapfile

- load_block_constraints -type FLOORPLAN -all_blocks

- Defining Routing Tracks

- create_track -layer METAL3 -space 0.5

- report_tracks

- remove_track -all

- Flipping the First Row in the Floorplan

- initialize_floorplan -flip_first_row false /:By default

- initialize_floorplan -flip_first_row true

- Updating the Floorplanning Without Disturbing Specified Object Types

- initialize_floorplan -keep_placement {all} /

- -keep_boundary

- -keep_pg_route

- -keep_detail_route

- -keep_placement {block}

- -keep_placement {io}

- -keep_placement {macro}

- -keep_placement {std_cell}

- -keep_placement {physical_only}

- cells can be identified

- get_cells -filter "design_type==physical_only

- get_cells -filter "design_type==fill

- Reporting Floorplan Information

- report_design –floorplan

- report_design –all

- Macro

add_array_to_macro_group     68

add_pins_to_virtual_connection . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . .103

add_macro_to_group . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 100

add_to_edit_group . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 137

add_to_group . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 139

add_to_io_guide . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . .140

add_to_io_ring . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . .142

- Validating Floorplan Rules

- Floorplan Area Rules

- Set_floorplan_area_rules –name area_rule_1 –object_types std_cell_area –min 100

- Floorplan Enclosure Rules

- set_floorplan_enclosure_rules -name enclosure_rule_1 - from_object_types routing_blockage -layers METAL -to_lib_cells $lib_cells -follow_rotations -sides all -must_enclose -min 5.0

- Floorplan Halo Rules

- set_floorplan_halo_rules -name halo_rule_1 - from_object_types routing_blockage -layers METAL2 -to_object_types hard_macro -sides all -type outer -must_enclose -min 3.0

- Floorplan Spacing Rules

- set_floorplan_spacing_rules -name spacing_rule_1 -from_lib_cells $lib_cells -follow_rotations -to_object_types std_cell_area -directions vertical -min_parallel_run_length -1.0 -step 2.0 -offset 13.0

- Floorplan Width Rules

- set_floorplan_width_rules -name width_rule_1 - object_types std_cell_area -type simple -direction horizontal -step 8.0 -offset 40.0

- IO Placement:

- Create I/O Rings

- create_io_ring -name outer_ring -corner_height 300

- get_io_guides

- create_io_guide -name "ring_bottom" -line {{3380 0} 3000} -side bottom

- create_io_guide -name "ring_right" -line {{3680 3380} 3000} - side right

- get_io_rings

- create_io_ring -name inner -inside outer -offset 500

- Create_pin_guide

- create_pin_guide -boundary {{332.7280 889.4720} {449.9640 899.4720}} -name input [get_ports -filter "direction == in"] -pin_spacing 1

- create_pin_guide -boundary {{332.7280 889.4720} {449.9640 899.4720}} -name output [get_ports -filter "direction == out"] -pin_spacing 1

- Create_port_pins

- place_pins-ports [get_ports -filter "direction == in"]

- place_pins-ports [get_ports -filter "direction == out"]

- Pins_checking

- Check_pin_placement

- To create voltage area:

- create_voltage_area -region {{718.3120 3.0000} {913.4800 317.3360}}                           -power_domain PD_RISC_CORE

- To create keepout_margin:

- create_keepout_margin {cell_name} -outer {3 3 3 3}

- To place macros :

- colour by hierarchy and initialize and/or shape_blocks

POWER PLAN SCRIPT

- Read the UPF file : load_upf

- To verify the UPF consistency and identify PG conflicts : resolve_pg_nets –check_only

- Report the associations made for the multivoltage cells : report_mv_path

- create_supply_net VSS

- create_supply_net VDD

- create_supply_net VDDH

- create_supply_set ss_high -function {power VDDH} -function {ground VSS}

- create_supply_set ss_low  -function {power VDD}  -function {ground ss_high.ground}

- create_power_domain PD_ORCA_TOP  -supply {primary ss_low}

- create_power_domain PD_RISC_CORE -elements I_RISC_CORE -supply {primary ss_high}

- create_supply_port VDD -domain PD_ORCA_TOP -direction in

- create_supply_port VSS -domain PD_ORCA_TOP -direction in

- create_supply_port VDDH -domain PD_ORCA_TOP -direction in

- The connect_pg_net command operates in two modes:

- Automatic :

- connect_pg_net -automatic

- Manual :

- connect_supply_net VSS  -ports VSS

- connect_supply_net VDD  -ports VDD

- connect_supply_net VDDH -ports VDDH

- set_level_shifter risc_ls_in -domain PD_RISC_CORE -applies_to inputs -rule low_to_high -location self

- set_level_shifter risc_ls_out -domain PD_RISC_CORE -applies_to outputs -rule high_to_low -location parent

- set_design_attributes -elements {.} -attribute correlated_supply_group {*}

- add_power_state -supply ss_high -state ON  { -supply_expr { power == {FULL_ON 0.95 0.95 1.16} && ground == {FULL_ON 0.0} } }

- add_power_state -supply ss_low  -state ON  { -supply_expr { power == {FULL_ON 0.75 0.75 0.95} && ground == {FULL_ON 0.0} } }

- create_power_state_group PST

- add_power_state -group PST -state RUN   {-logic_expr { ss_low==ON && ss_high==ON  }}

- Merging Voltage Area Shapes :

- When you use the create_voltage_area -merge_regions command, the merges the shapes specified with the -region option.

- When you use the set_voltage_area -merge_regions command, the tool merges all existing shapes of the specified voltage area.

- Guard Bands :

- Guard bands define hard keepout margins surrounding the voltage areas in which no cells, including level shifters and isolation cells, can be placed. The guard bands guarantee that the cells in different voltage areas are separated so that power planning does not introduce shorts.

- create_voltage_area -guard_band <value>

- Modifying Voltage Areas :

- To change the power domains associated with a voltage area

- set_voltage_area -add_power_domains

- set_voltage_area -remove_power_domains

- Change the voltage area name set_voltage_area –name

- To change the voltage area region to add shapes to a voltage area create_voltage_area_shape

- To remove shapes from a voltage area remove_voltage_area_shapes

- Controlling Physical-Feedthrough Nets in Voltage Areas :

- To prevent physical-feedthrough nets in a voltage area

- create_voltage_area_rule -allow_pass_through false

- Controlling the Placement of Multivoltage Cells

- To reduce the length of multivoltage nets, enable advanced multivoltage cell placement by setting the place.coarse.enable_advanced_mv_cell_placement -value true

PrePlacement

- Boundary cell:

- create_boundary_cells -left_boundary_cell saed32_rvt|saed32_rvt_std/DCAP_RVT   -right_boundary_cell saed32_rvt|saed32_rvt_std/DCAP_RVT -top_boundary_cells saed32_rvt|saed32_rvt_std/DCAP_RVT -bottom_boundary_cells saed32_rvt|saed32_rvt_std/DCAP_RVT

- check_boundary_cells

- Tap cell:

- create_tap_cells -lib_cell saed32_rvt|saed32_rvt_std/DCAP_RVT -distance 30 -pattern stagger -prefix TAP-skip_fixed_cell

- check_tap_cells

Placement

Scan DEF without

set_app_options -name place.coarse.continue_on_missing_scandef-value true

Scan def with

read_def path_info

Standalone Placement

- create_placement –congestion –congestion_effort high –increamnetal -     buffer_aware_timing_driven

- create_placement –congestion_driven_restructuring –congestion_effort high –increamnetal -buffer_aware_timing_driven

- create_placement -timing_driven -increamental -buffering_aware_timing_driven

- set_lib_cell_purpose –include optimization {tech_lib/buf3x tech_lib/buf8x}

Legalization:

- legalize_placement

- check_legality

Place_opt:

Set_app_options

- Congestion:

set_app_options -name plan.place.congestion_driven_mode -value both

- Timing:

set_app_options -name plan.place.timing_driven_mode -value both

- IR enable

set_app_options -name place.coarse.ir_enable_ir -value true

- Clock Tree for Placement and Optimization:

- Set_app_options –name place_opt.flow.trial_clock_tree –value true

- Set_app_options –name place_opt.flow.clock_aware_placement –value true

- Set_app_options –name place_opt.flow.optimize_icgs_critical_range –value 0.0 to 1

- Latencies for Clock Gates:

- set_clock_latency -min

- set_attribute [get_pins {ICG21/CK}] dont_estimate_clock_latency true

- Enabling Global Route Based High-Fanout Synthesis:

- set_app_options -name place_opt.initial_drc.global_route_based -value 1

- set_app_options -name place_opt.initial_place.two_pass -value 1

- set_app_options -name opt.buffering.enable_mv_rebuffering -value true

- set_app_options -name opt.buffering.enable_rebuffering -value true

- Changing the Congestion Effort:

- set_app_options -name place_opt.place.congestion_effort -value medium

- Nondefault Routing Rules for Critical Nets:

- set_app_options –name place_opt.flow.optimize_ndr –value ture

- Enabling Path Optimization:

- Set_app_options –name place_opt.flow.do_path_opt –value true

- Performing IR-Drop-Aware Placement:

- source redhawk_setup.tcl

- analyze_rail -voltage_drop static -nets {VDD VSS}

- set_app_options –name place.coarse.ir_drop_aware –value true

- set_app_options -name power.leakage_mode -value average

- Synopsys physical guidance:

- set_app_options -name place_opt.flow.do_spg -value true

- read_verilog design_spg.v

- source timing_const.tcl

- load_upf design.upf

- read_def -add_def_only_objects {cells}

- Integrated Multibit Register Optimization:

- Set_app_options –name place_opt.flow.enable_multibit_banking –value true

- Set_app_options –name place_opt.flow.enable_multibit_debanking –value true

- Set_app_options –name multibit.banking.across_equivalent_icg –value true

- Set_app_options –name place_opt.flow.enable_multibit_rewiring –value true

- set_multibit_options -exclude [get_cells "reg[1]"]

- Set_app_options –name multibit.common.ignore_sizeonly_cells –value true

- report_multibit

- split_multibit

- split_multibit -slack_threshold -1

- Magnet Placement:

- magnet_placement Cell2 -cells {Cell6 Cell7 Cell8}

- magnet_placement –get_collection

- Refining Placement:

- refine_placement –effort medium

- refine_placement –coordinates –congestion_effort medium

- refine_placement -perturbation_level medium

Commands

place_opt -from initial_place -to initial_place

place_opt -from initial_drc -to initial_drc

place_opt -from initial_opto -to initial_opto

place_opt -from final_place -to final_place

place_opt -from final_opto -to final_opto

CTS : APP_OPTIONS

- set_app_options -name abstract.keep_constrained_objects -value true

- set_app_options -name ccd.dps.auto_detect_cts_options  -value true

- set_app_options -name ccd.respect_cts_fixed_balance_pins  -value true

- set_app_options -name cts.balance_groups.balance_by_source_latency   -value true

- set_app_options -name cts.balance_groups.honor_source_latency    -value true

- set_app_options -name cts.buffering.reduce_clock_level_effort     -value true

- set_app_options -name cts.buffering.reduce_clock_level_effort  -value true

- set_app_options -name cts.common.auto_exception_macro_balance_point    -value true

- set_app_options -name cts.common.default_max_transition   -value 0.5

- set_app_options -name cts.common.derive_icg_and_register_references     -value true

- set_app_options -name cts.common.derive_icg_and_register_references     -value true

- set_app_options -name cts.common.directory_for_cts_generated_files     -value true

- set_app_options -name cts.common.enable_auto_exceptions  -value true

- set_app_options -name cts.common.enable_auto_skew_target_for_local_skew   -value true

- set_app_options -name cts.common.enable_dirty_design_mode   -value true

- set_app_options -name cts.common.enable_low_power    -value true

- set_app_options -name cts.common.ignore_drc_only_scenario     -value true

- set_app_options -name cts.common.max_fanout      -value true

- set_app_options -name cts.common.max_fanout  -value true

- set_app_options -name cts.common.max_fanout  -value 10000

- set_app_options -name cts.common.max_net_length  -value      3.40282e+3

- set_app_options -name cts.common.power_aware_pruning -value true

- set_app_options -name cts.common.preserve_bus_ports -value true

- set_app_options -name cts.common.skip_cts_clocks  -value true

- set_app_options -name cts.compile.cg_timing_aware   -value true

- set_app_options -name cts.compile.enable_cell_relocation  -value true

- set_app_options -name cts.compile.enable_cell_relocation  -value true

- set_app_options -name cts.compile.enable_cell_relocation  -value false

- set_app_options -name cts.compile.enable_cell_relocation -value false

- set_app_options -name cts.compile.enable_cell_relocation -value all

- set_app_options -name cts.compile.enable_global_route  -value all

- set_app_options -name cts.compile.enable_global_route -value all

- set_app_options -name cts.compile.enable_global_route -value true

- set_app_options -name cts.compile.enable_global_route

- set_app_options -name cts.compile.enable_local_skew -value true

- set_app_options -name cts.compile.enable_local_skew -value true

- set_app_options -name cts.compile.fix_clock_tree_sinks  -value true

- set_app_options -name cts.compile.global_route_aware_buffering   -value true

- set_app_options -name cts.compile.power_opt_mode   -value true

- set_app_options -name cts.compile.power_opt_mode   -value true

- set_app_options -name cts.compile.power_opt_mode   -value none

- set_app_options -name cts.compile.power_opt_wirelength_effort    -value low

- set_app_options -name cts.compile.primary_corner    -value true

- set_app_options -name cts.compile.report_latency_bottleneck     -value true

- set_app_options -name cts.compile.size_pre_existing_cell_to_cts_references -value true

- set_app_options -name cts.compile.timing_driven_sink_transition  -value true

- set_app_options -name cts.compile.topological_ndr   -value true

- set_app_options -name cts.icg.timing_aware   -value true

49.  set_app_options -name cts.multisource.cell_em_aware    -value true

50.  set_app_options -name ts.multisource.enable_full_mv_support     -value true

51.  set_app_options -name cts.multisource.enable_full_mv_support     -value true

52.  set_app_options -name cts.multisource.enable_multi_corner_support      -value true

53.  set_app_options -cts.multisource.cto_toggle_rate_fanout_threshold            -value 50

54.  set_app_options -name         cts.multisource.cto_toggle_rate_fanout_threshold      -value 50

55.  set_app_options -name cts.multisource.cto_toggle_rate_scale      -value 3

56.  set_app_options -name cts.multisource.enable_pin_accessibility_for_global_clock_trees      -value 3

57.  set_app_options -name cts.multisource.enable_pin_accessibility_for_global_clock_trees      -value true

58.  set_app_options -name cts.multisource.enable_subtree_synthesis_aware_ccd        -value true

59.  set_app_options -name cts.multisource.enable_via_ladder_insertion_on_tap_output_pins   -value true

60.  set_app_options -name cts.multisource.tap_synthesis_route_based_estimation   -value true

61.  set_app_options -name cts.icg.latency_driven_cloning   -value true

62.  set_app_options -name cts.routing.global_route_topology    -value true

63. set_app_options -name cts.routing.global_route_topology    -value 0

64. set_app_options -name cts.routing.global_route_topology    -value automatic

65.  set_app_options -name cts.placement.cell_spacing_rule_style      -value true

66.  set_app_options -name cts.placement.cell_spacing_rule_style      -value maximum

67.  set_app_options -name place.coarse.irdp_cts_spacing      -value true

ROUTING

Routing 1ST stage is GLOBAL ROUTING:

- Set_app_options -name route.global.effort_level -value high/medium/low

- Set_app_options -name route.global.crosstalk_driven -value true

- Set_app_option -name route.global.timing_driven -value true

- Set_app_options -list {place.legalize.avoid_backside_pins_under_preroute_layers {bmetal1 bmetal2}}

- Set_app_options -name route.global.advance_node_timing_driven_effort -value high/low/medium

- Set_app_options -name route.global.double_pattern_utilization_by_layer_name -value { {M1 85} {M2 85} {M5 70} }

- Set_app_options -name route.global.export_soft_congestion_maps -value true/false

- Set_app_options -name route.global.macro_area_iterations -value 3

- Set_app_options -name route.global.macro_area_track_utilization -value 80

- Set_app_options -name route.global.macro_boundary_track_utilization -value 75

- Set_app_options -name route.global.net_type_based_blockage_boundary_gcell_enhancement -value true

- Set_app_options -name route.global.power_aware_routing -value true

- Set_app_options -name route.global.voltage_area_corner_track_utilization -value 80

The command for global route is:

- Route_global_effort_level -value high

- The global routing involves the approximate paths for interconnections across the chip without specifying the exact global routing.

- The effort_level controls the level applied during global routing.

Routing 2nd stage is TRACK ASSIGNMENT:

- Set_app_options -name route.track.allow_wire_outside_gcell -vlaue true/false

- Set_app_options -name route.track.crosstalk_driven -value false/true

- Set_app_options -name route.track.timing_driven -value true

The command for track assignment is:

- Route_track

- Assign nets to specific routing tracks within the global routing region.

- Ensures that the routing resources(tracks) are used efficiently while avoiding conflicts and minimizing congestion.

- A more refined routing plan with nets assigned to specific tracks.

Routing 3rd stage is DETAILED ROUTING:

- Set_app_options -name route.detail.allow_default_rule_nets_via_ladder_lower_layer_connection -value true

- Set_app_options -name route.detail.antenna -value false/true

- Set_app_options -name route.detail.antenna_fixing_preference -value use_diodes

- Set_app_options -name route.detail.antenna_fixing_preference -value hop_layers

- Set_app_options -name route.detail.antenna_on_iteration -value 5

- Set_app_options -name route.detail.check_antenna_for_open_nets -value true

- Set_app_options -name route.detail.check_antenna_on_pg -value true

The command for detail route is:

- Route_detail -max_number -iteration 20

- The detail routing which creates the exact physical wiring for each net while adhering to design rules and constraints.

- Max_number_iterations: each iteration involves the tool attempting to fix DRC’S shorts and opens by rerouting or adjusting the wiring

- All issues are resolved before reaching the maximum number of iterations.

(Or)

- Route_opt